Homework Set 1 – ISA and System Evaluation

Due Date: Friday, Oct 23, 2026

Problem 1: Short Questions. [25 points]

Part A: Geometric Mean. [4 points]

Prove the following property of geometric means: for two sets of positive values xi and yi, i=1, 2, …, n,

Geomeani=1nyixi=Geomeani=1nyiGeomeani=1nxi

With this property, we can use geometric means to average the performance speedup. Say for a set of benchmark workloads Wi, i=1, 2, …, n, running on three different systems A, B, and C, assume the execution times are ai, bi, ci, i=1, 2, …, n, respectively. The average speedup of B over A is SB/A=Geomeani=1naibi, and the average speedup of C over A is SC/A=Geomeani=1naici. Prove that the average speedup of C over B is SC/B=SC/ASB/A, i.e., the relative speedup is irrelevant of the reference.

Part B: Amdahl’s Law vs. Gustafson's Law. [6 points]

1. [4 points] Amdahl’s Law applies to the scenario with a fixed amount of workloads. Now we derive Gustafson's Law, which deals with another scenario where the total amount of workloads can be increased when more processors are available.

Assume we have an infinite number of jobs, e.g., an online web search service that continuously serves users’ requests. Each processor can finish η jobs in unit time. We would like to derive the ratio of the throughput with N processors and that with a single processor. To do this, Gustafson’s Law uses a fixed time T, and let the processor(s) finish as many jobs as possible in each case. Regardless of how many processors used, only in a constant fraction f of the time T we can fully utilize all processors in parallel, while the rest time is serial and only makes use of a single processor. Derive the speedup in terms of throughput of N processors over one processor.

Hint: your final equation should only contain N and f, and when f→1, the speedup →N.

2. [2 points] As you may wonder, Amdahl’s Law can be used to describe the strong scaling condition (fixed total workloads), and Gustafson’s Law can be used to describe the weak scaling condition (fixed total time, so total workloads increases with more processors). Assume the parallel portions f (portion of workload in Amdahl’s Law, and portion of time in Gustafson’s Law) are equal in the two cases, prove that strong scaling has lower speedup than weak scaling with the same number of processors N.

Part C: Performance. [15 points]

Assume for arithmetic, load/store, and branch instructions that a processor has cycles-per-instruction (CPIs) of 1, 10, and 5, respectively. Also assume that on a single processor a program requires the execution of 2.56 billion arithmetic instructions, 1.28 billion load/store instructions, and 256 million branch instructions. Assume that each processor has a 4 GHz clock frequency.

Assume that, as the program is parallelized to run over multiple cores, the number of arithmetic and load/store instructions per processor is divided by 0.8 x p (where p > 1 is the number of processors) but the number of branch instructions per processor remains the same.

1. [10 points] Find the total execution time for this program on 1, 2, 4, and 8 processors, and show the relative speedup of the 2, 4, and 8 processor result relative to the single processor result.

2. [5 points] Now we would like to design a new processor with better load/store performance, so that the performance of a single processor of this new type could match the performance of four processors of the original type (as you have calculated above). How much should the CPI of load/store instructions in the new processor be to achieve this goal? Assume the other instructions’ CPI values are the same as original.

Problem 2: RISC-V to C. [10 points]

1. [5 points] For the RISC-V assembly instructions below, what is the corresponding single C statement? Assume that variables i and j are assigned to registers s0 and s1, respectively. The base address of the arrays A and B are in registers s6 and s7, respectively. Each element in the arrays is 4B. Please briefly explain your approach.

1    slli  t0, s0, 2

2    add   t0, s6, t0

3    slli  t1, s1, 2

4    add   t1, s7, t1

5    lw    s2, 0(t0)

6    addi  t2, t0, 16

7    lw    t0, 0(t2)

8    add   t0, t0, s2

9    addi  t1, t1, 4

10   sw    t0, 0(t1)

2. [5 points] For the RISC-V assembly instructions above, rewrite the assembly code to minimize the number of RISC-V instructions (if possible) needed to carry out the same function.

Problem 3: C to RISC-V. [25 points]

1. [8 points] Expand the pseudo-instructions shown below into actual RISC-V assembly instructions. Use as few instructions and as few extra registers as possible in your answers. If you use more than needed, you only get partial credits.

|  | Pseudo-instruction | What it accomplishes |
| --- | --- | --- |
| 1 | not rd, rs | rd = ~rs # bit-wise invert |
| 2 | li rd, BIG | rd = BIG # load 32-bit BIG to rd |
| 3 | seqz rd, rs | rd = 1 if (rs == 0), 0 otherwise |
| 4 | snez rd, rs | rd = 1 if (rs != 0), 0 otherwise |
| 5 | ble rs1, rs2, L | if (rs1 <= rs2) goto L |
| 6 | call LFAR | ra = pc + 4, goto LFAR # call a far-away procedure |

In the above table, BIG is a 32-bit immediate number. LFAR is a label for a far-away procedure, which means the offset between its address and the current PC is large and needs 32 bits to represent. You can directly use this offset value in your implementation as it is a compile-time constant.

Hint: Pay attention to the most significant bit when you split these numbers, e.g., BIG[11] and offset[11]. You could use expressions involving only compile-time constants in your solutions.

|  | Pseudo-instruction | Implementation with base instruction(s) |
| --- | --- | --- |
| 1 | not rd, rs |  |
| 2 | li rd, BIG |  |
| 3 | seqz rd, rs |  |
| 4 | snez rd, rs |  |
| 5 | ble rs1, rs2, L |  |
| 6 | call LFAR |  |

2. [9 points] Implement the following C code using RISC-V assembly. The code realizes a binary search in a sorted array of words. Use as few instructions as possible, i.e. make the program as short as possible. If you use more than needed, you only get partial credits. You are allowed to use the pseudo-instructions we have introduced in class.

// inputs are A, n, v; output is i

int l = 0;

int r = n - 1;

int i = -1;

while (l <= r) {

int m = (l + r) / 2;

if (A[m] < v) l = m + 1;

else if (A[m] > v) r = m – 1;

else { i = m; break; }

}

The base address of the array is stored in a0, and its length in words is stored in a1. You can assume the array is not empty, i.e., n > 0. The target value to search for is in a2. I.e., a0 = A, a1 = n, a2 = v. Return the output value i in s0. Each word is 4 bytes. All registers except zero have initially undefined values and do not need to be restored.

3. [8 points] Implement the following code that involves function calls using RISC-V assembly. The function print_and_sum(n) prints every value from 1 to n, and returns the sum of these values. It is called from the main() function, and itself calls the C library function putchar(). You do not have visibility to the main() and putchar() function implementations so you cannot know which registers they may use. You can assume the assembly of putchar() starts from a location marked with the label putchar.

You need to implement the complete function of print_and_sum() following the RISC-V calling convention. For example, all variables must be mapped to registers according to the RISC-V calling convention. You do not need to optimize code length for performance.

int print_and_sum(int n) {

int sum = 0;

int i = 0;

while(i < n) {

i++;

putchar(i);

sum = sum + i;

}

return sum;

}

Problem 4: Assembly Code Analysis. [40 points]

The following code processes an array and produces two values in registers s2 and s3. Assume that the array consists of 300 words (indexed from 0 to 299). Its base address is stored in a0, and its length in words is stored in a1 (i.e., a1 = 300). Each word is 4 bytes.

begin:

1     addi  s3, zero, -1

2     addi  s5, zero, -1

3     add   a2, a1, a1

4     add   a2, a2, a2

5     add   t0, zero, zero

6     add   s4, zero, zero

7     add   t4, a0, t0

8     lw    t4, 0(t4)

outer:

9     add   t5, zero, zero

10    add   t1, zero, zero

inner:

11    slli  t2, t1, 2

12    add   t3, a0, t2

13    lw    t3, 0(t3)

14    bne   t3, t4, skip

15    addi  t5, t5, 1

skip:

16    addi  t1, t1, 1

17    bne   t1, a1, inner

18    sltu  t2, s5, t5

19    bne   t2, zero, elsepart

20    add   s3, s5, zero

21    add   s2, s4, zero

22    add   s5, t5, zero

23    add   s4, t4, zero

24    jal   zero, next

elsepart:

25    sltu  t2, s3, t5

26    bne   t2, zero, next

27    add   s3, t5, zero

28    add   s2, t4, zero

next:

29    addi  t0, t0, 4

30    bne   t0, a2, check

31    jal   zero, exit

check:

32    add   t4, a0, t0

33    lw    t4, 0(t4)

34    bne   t4, s4, outer

35    jal   zero, next

exit:

1. [6 points] Describe in one sentence what this code does. Specifically, what will be returned in s2 and s3?

2. [12 points] What is the total number of instructions that are executed in this piece of code in the worst case? Explain what the worst case is. Briefly explain the number of instructions executed in each loop (e.g. There are 8 instructions that are executed once before loop outer, there are 7 instructions in loop inner that are executed 10 times, etc.).

3. [12 points] Assume that the code fragment is run on a machine with a 1.8 GHz clock that requires the following number of cycles for each instruction:

| Instruction | Cycles |
| --- | --- |
| sltu, slli, jal | 1 |
| bne, add, addi | 2 |
| lw | 3 |

In the worst case, how many seconds will it take to execute this code? You can make reasonable approximation to simplify the calculation.

4. [10 points] Consider the inner loop counter variable, t1. On each iteration, this variable is tested and incremented (line 16 – 17):

addi	t1,	t1,	1

bne	t1,	a1,	inner

Consider an ISA that is similar to RISC-V except that a new instruction, rpt, is added to assist with loop iteration:

rpt	t1, 	a1, 	inner

The new instruction replaces the above two instructions and takes 2 cycles to execute. The rpt instruction increments the first argument, compares with the second argument, and performs a branch if the results are not equal. Unfortunately, to accommodate this new instruction, the CPU clock cycle time is increased by 25%.

In the worst case, how many seconds will it take to execute the code listed above using the new ISA? How much faster or slower compared to the original RISC-V?